PRODUCT SPECIFICATION – QuietZone Acoustic Sealant

Section 09 80 00 Acoustical Treatment

Section 09 85 00 Sound transmission Control Materials

Part 1 – GENERAL

  - Summary

Premium siliconized acrylic-based sealant to seal openings in walls, floors, ceilings,

doors and windows.

  - Related Products

- Section 09 85 00 -QuietZone Acoustic Batt Insulation

- Section 09 85 00 -QuietZone Acoustic Floormat

- Section 09 85 00 -QuietZone Acoustic Wall Framing

- Section 09 77 40 -QuietZone Solserene Fabric Ceiling System

- Section 09 54 00 –QuietZone Acoustyle Wood Ceiling System

  - References

- American Society for Testing and Materials (ASTM)

  - E84 Test Method for Surface Burning Characteristics

  - C834, Type P, Grade – 18 degrees C.

  - C920, Class 25

  - Exceeds Federal Specification TT-S-00230C

  - Submittals

- Product Data:  Submit Owens Corning product literature, installation instructions

for specified sealant.

  - Delivery, Storage and Handling

- Protect insulation from physical damage.  Do not allow to freeze in tube.

B.   Comply with manufacturer’s recommendations for handling, storage and protection

during installation.

C.   Label for product to include material name, production date and/or product code.

  - Limitations

A.  Do not use as spackling

B.  Apply at temperatures above 40 degrees F.

C.  Not recommended for holes or gaps larger than ½ inch.

D.  Not recommended for aquariums, stoves, food areas, marble or underwater.

Part 2 – PRODUCTS

2.01 Manufacturer

A.  Owens Corning

2.02 QuietZone Acoustic Sealant

A.  Available in 10.1 ounce and 29 ounce tubes

B.  Non-hardening, premium siliconized acrylic sealant for interior and exterior use.

C.  Paintable, water clean-up.

Part 3 – EXECUTION

3.01 Inspection

A.  Examine the areas and conditions under which work of this section will be installed.

Verify that adjacent materials are dry and ready.  Verify that electrical and

mechanical sevices within walls have been inspected and tested.  Verify that project

drawings comply with installation requirements.

B.  Provide written report listing conditions detrimental to the performance of work in

this section.  Do not proceed with installation until unsatisfactory conditions have

been resolved.

3.02 Installation

A.  Comply with manufacturer’s instruction for particular conditions of installation

in each case.

B.  Apply to a clean, dry surface

C.  Cut tip to suggested bead size.  Apply at a 45 degree angle then “tool” bead into place after 3-5 minutes.

D.  Superior adhesion to wood, glass, metal and masonry.

3.04 Material Storage and Protection

Protect sealant tubes from damage and from freezing prior to installation.

END OF SECTION